STM32
SESSION

A Practical Instructor's Guide

CubeMX & IOC • HAL & Registers • Timers • DMA • ADC • UART/SPI/I²C • Sensor Drivers • FreeRTOS

SESSION GOAL Help students make sound embedded-engineering decisions, configure STM32 peripherals, implement and debug them in STM32CubeIDE, and connect individual topics into a working data-acquisition pipeline.

Designed for a six-hour evening session, with core explanations, configuration walkthroughs, practical demonstrations, common mistakes, and student tasks.

Board note: examples are intentionally family-aware. Register names, clock trees, DMA routing, HAL APIs, and available peripherals vary by STM32 family. Confirm every example against the exact MCU reference manual and datasheet.

# How to Use This Guide

Teach each topic using the same sequence: problem → concept → engineering choice → CubeMX configuration → code → test → failure modes. Avoid presenting peripheral names as a vocabulary list without showing what problem each peripheral solves.

| Session block | Suggested time | Outcome |
| --- | --- | --- |
| 1. Development workflow & CubeMX | 35–40 min | Students can explain the IOC-to-firmware workflow and distinguish HAL from direct register access. |
| 2. Interrupts, DMA & timers | 50–60 min | Students can choose an execution/data-transfer strategy and understand timer-triggered sampling. |
| 3. UART, SPI & I²C | 35–40 min | Students can select a bus based on wiring, speed, topology, and device requirements. |
| 4. Sensors, ADC & acquisition | 35–40 min | Students understand sampling, scaling, timing, and buffering. |
| 5. Reusable sensor drivers | 30–35 min | Students can separate sensor logic from application code. |
| 6. RTOS fundamentals | 25–30 min | Students can decide whether an RTOS is justified and sketch a task/queue design. |
| Integrated example, questions & buffer | Remaining time | Connect the session topics and leave time for troubleshooting. |

INSTRUCTOR RULE For every topic, answer six questions: What is it? Why does it exist? When should I use it? How do I configure it? How do I implement it? What mistakes should I avoid?

## Six-hour schedule example

| Time | Activity |
| --- | --- |
| 18:00–18:30 | Arrival, toolchain checks, project files, hardware check |
| 18:30–19:10 | Part 1 — CubeIDE, CubeMX, IOC, clocks, HAL/registers |
| 19:10–20:00 | Part 2 — polling, interrupts, DMA, timers, ADC trigger chain |
| 20:00–20:30 | Prayer/rest break; adjust to local prayer times |
| 20:30–21:10 | Part 3 — UART, SPI, I²C and protocol selection |
| 21:10–21:45 | Part 4 — sensors, ADC and real-time data acquisition |
| 21:45–22:20 | Part 5 — reusable drivers |
| 22:20–22:45 | Part 6 — RTOS concepts |
| 22:45–23:15 | Integrated example / student task mapping |
| 23:15–00:00 | Questions, debugging, optional deeper examples |

# PART 1 — STM32 Development Workflow

Suggested duration: 35–40 minutes.

## 1.1 CubeIDE, CubeMX and the IOC file

| Item | What it does | What it does not do |
| --- | --- | --- |
| STM32CubeMX | Configures pins, clocks, peripherals, middleware and code-generation settings. | It does not write all application behavior for you. |
| STM32CubeIDE | Provides project editing, build/compile, flashing and debugging workflows. | It does not remove the need to understand hardware and generated code. |
| .ioc file | Stores the CubeMX project configuration: MCU/board, pin mapping, clock tree, peripheral settings and generation choices. | It is not the compiled firmware or a replacement for source control. |
| Generated initialization code | Initializes configured clocks, GPIO and peripheral handles/settings. | It is not a complete application; user logic still needs to be implemented. |

Typical workflow:

Create a project and select the exact MCU or board.

Configure pin assignments and resolve pin conflicts.

Configure the clock tree and verify all clock limits.

Enable peripherals and set their parameters.

Configure NVIC, DMA, middleware and code-generation options as required.

Save the .ioc file and generate code.

Review generated initialization and add application logic inside user-code regions or separate application files.

Build, flash, test, and debug. Keep the .ioc and source files together in version control.

CODE-GENERATION WARNING Regenerating code can overwrite edits made outside protected user-code sections. Keep application logic in user sections or, preferably, separate source/header files. Review generated diffs after changing the .ioc.

## 1.2 CubeMX terminology glossary

| Term | Meaning and teaching note |
| --- | --- |
| ADC | Analog-to-Digital Converter; samples an analog voltage and returns a digital code. |
| BSP | Board Support Package; board-specific setup and helper code. |
| CAN / FDCAN | Controller Area Network interfaces used for robust message-based communication; FDCAN supports CAN FD where available. |
| CRC | Cyclic Redundancy Check hardware; calculates/checks a checksum-like value for error detection. |
| DAC | Digital-to-Analog Converter; produces an analog output from a digital code. |
| DMA | Direct Memory Access; moves data between supported peripheral/memory endpoints with reduced CPU copying. |
| EXTI | External Interrupt/Event controller; detects configured GPIO or internal events and can request CPU attention. |
| FPU | Floating-Point Unit; hardware support for floating-point arithmetic on supported cores. |
| GPIO | General-Purpose Input/Output; digital input/output pins with modes, pulls, output type and speed settings. |
| I²C | Two-wire synchronous addressed bus using SDA and SCL, typically with pull-up resistors. |
| IWDG | Independent Watchdog; resets the MCU if it is not refreshed in time, using an independent clock source on supported devices. |
| NVIC | Nested Vectored Interrupt Controller; enables/prioritizes CPU interrupts and dispatches handlers. |
| PWR | Power control; voltage scaling, low-power modes and power-related configuration, depending on MCU. |
| RCC | Reset and Clock Control; configures oscillators, PLLs, bus clocks, peripheral clocks and resets. |
| RTC | Real-Time Clock; calendar/timekeeping or low-power time base, usually with a suitable clock source. |
| SAI / I²S | Audio interfaces for digital audio streams, available on selected MCUs. |
| SPI | Synchronous serial interface using clock and data lines; often selected for high-throughput peripheral links. |
| SYS / System Core | CubeMX grouping for core/system configuration such as debug, SysTick/time base and related system settings; exact options vary. |
| TIM | Timer peripheral; supports periodic timing, PWM, input capture, output compare and trigger generation, depending on timer type. |
| UART / USART | Serial communication peripheral; UART is asynchronous, while USART may support synchronous modes too. |
| USB | USB device/host/OTG functionality on supported MCUs; requires correct clocking and board hardware. |
| WWDG | Window Watchdog; requires refresh within a configured time window; early or late refresh can cause a reset. |
| FreeRTOS | Optional real-time operating system middleware for tasks, scheduling, queues, semaphores and related services. |
| Middleware | Reusable software stacks such as USB, file systems, networking or RTOS components. |
| Pinout & Configuration | CubeMX view for pin mapping, peripheral enablement and settings. |
| Clock Configuration | Clock-tree editor for oscillator, PLL and bus clock setup. |
| Peripheral Parameters | Settings for a peripheral instance, such as baud rate, data width, trigger, sampling time or mode. |
| NVIC Settings | Interrupt enable and priority settings for supported interrupt sources. |
| DMA Settings | DMA request/channel/stream, direction, mode, increment, alignment and priority settings; exact fields vary by family. |
| GPIO Settings | Pin mode, pull-up/down, output type, speed and alternate function where applicable. |

## 1.3 How to configure clocks using the IOC file

Clock configuration determines how fast the core, buses and peripherals operate. Incorrect clocks can break UART baud rates, timer periods, USB requirements, ADC timing or power limits.

Open the .ioc file and go to Clock Configuration.

Identify the available clock sources: internal oscillators (for example HSI/LSI), external oscillator inputs (HSE/LSE) and PLL options. Availability depends on the MCU and board.

Choose the system clock source and PLL settings if needed.

Check SYSCLK, HCLK, APB bus clocks, peripheral kernel clocks and flash latency against the device's limits.

Check the clock feeding the peripheral you plan to use. Timer clocks may not equal the CPU clock; on many STM32 families the APB prescaler affects timer clocking.

Resolve CubeMX warnings, save the .ioc file, regenerate code, then verify the generated clock initialization.

Measure a known output or inspect clock registers when debugging; do not rely only on a diagram.

CLOCK MATH EXAMPLE If a timer input clock is 72 MHz and PSC = 71, the counter clock is 72 MHz / (71 + 1) = 1 MHz. If ARR = 999, an update event occurs every 1,000 counter ticks, i.e. every 1 ms. This assumes the selected timer clock really is 72 MHz and ordinary up-counting.

## 1.4 Enabled peripherals and peripheral parameters

Enabling a peripheral tells CubeMX to allocate/configure the peripheral and usually assign its pins or clock dependencies. Parameters describe how that instance behaves.

| Peripheral | Example parameters to explain |
| --- | --- |
| GPIO | Input/output/alternate function/analog mode; pull; push-pull/open-drain; output speed. |
| UART | Baud rate, word length, parity, stop bits, oversampling, flow control, TX/RX pins. |
| I²C | Timing/clock speed, addressing mode, own address, analog/digital filters where supported. |
| SPI | Controller/peripheral role, direction, data size, CPOL, CPHA, NSS, prescaler. |
| ADC | Resolution, channel/rank, sample time, trigger source, continuous mode, alignment, scan mode. |
| TIM | Prescaler, counter mode, period/ARR, clock division, PWM channel/polarity, trigger and interrupt settings. |
| DMA | Direction, peripheral/memory increment, data width, normal/circular mode, priority. |
| NVIC | Enable relevant IRQ and set priority/subpriority according to the MCU's priority model. |

## 1.5 HAL vs. register-level programming

HAL (Hardware Abstraction Layer) provides higher-level functions and common patterns. Register-level programming accesses peripheral registers directly. Both ultimately control the same hardware.

| Dimension | HAL | Register-level |
| --- | --- | --- |
| Abstraction | Function calls and handles hide many register details. | Code sets/clears register fields directly. |
| Development | Often quicker and easier to review for common tasks. | Can be concise but requires register and reference-manual knowledge. |
| Portability | Can ease movement within supported STM32 families, but APIs/configuration still differ. | Usually tightly coupled to the exact MCU/peripheral implementation. |
| Control | Convenient APIs and built-in status/error conventions. | Fine-grained control and precise ordering where needed. |
| Performance | Some calls add overhead; impact depends on the operation. | Can reduce overhead for selected operations, but incorrect code can be unsafe. |
| Best use | Most application-level peripheral work and maintainable team code. | Learning hardware, critical paths, unsupported details or justified optimization. |

HAL example — reset an output pin:

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_13, GPIO_PIN_RESET);

Illustrative direct BSRR write to reset PC13 on a compatible STM32 GPIO implementation:

GPIOC->BSRR = (1U << (13U + 16U));

Both examples assume PC13 exists on the selected MCU, GPIOC's clock is enabled, and the pin has been configured as an output. Register layouts and available pins vary; use the exact reference manual.

## Part 1 practical task

Configure a GPIO output and make an LED blink.

Identify the generated GPIO initialization function and clock-enable code.

Find the user-code region where application behavior belongs.

Change the blink period, build, flash and verify.

Ask students to explain which settings came from CubeMX and which statements they wrote.

# PART 2 — Polling, Interrupts, DMA and Timers

Suggested duration: 50–60 minutes. Emphasize the difference between event timing, CPU notification and data movement.

## 2.1 Polling vs. interrupts vs. DMA

| Approach | How it works | Use it when | Main caution |
| --- | --- | --- | --- |
| Polling | CPU repeatedly checks status or waits for a function to finish. | Simple, infrequent actions; quick prototypes; short bounded waits. | Can waste CPU time or delay other work if blocking. |
| Interrupt | A hardware event requests CPU service; an ISR/callback runs. | Events need prompt handling without continuously checking status. | Keep ISR work short; respect priority and shared-data rules. |
| DMA | A hardware controller transfers data between supported endpoints and memory. | Repeated or larger transfers, streaming, ADC blocks, UART reception/transmission. | Requires correct buffer ownership, transfer configuration and completion/error handling. |

KEY DISTINCTION ADC converts. A timer schedules/triggers. DMA transfers. Interrupts notify the CPU. Application code processes results. These functions complement one another rather than replace one another.

## 2.2 When should DMA be used?

Use DMA when data is frequent/continuous, blocks are large, CPU time matters, or per-item interrupts/polling would be expensive.

Consider interrupt or polling for a one-off, low-rate sensor read where simplicity matters more than throughput.

Use circular DMA for repeated streaming when supported and suitable; normal DMA is useful for finite blocks.

Handle half-transfer and transfer-complete notifications if processing a circular buffer in halves.

Ensure the application processes data before the DMA overwrites it.

On MCUs with data cache, observe cache coherency requirements for DMA buffers; cache handling is family/core-specific.

Check DMA request routing, memory accessibility, alignment, data widths, and transfer direction in the reference manual.

## 2.3 STM32 timers explained

A timer counts clock events. Depending on its type, it can generate periodic update events, PWM, input capture, output compare, encoder measurements, or trigger signals for other peripherals.

| Timer feature | What it is useful for |
| --- | --- |
| Base/update event | Periodic scheduling and timer interrupts. |
| PWM | Motor speed commands, LED dimming, servo-style control signals and power conversion where appropriate. |
| Input capture | Measure the time between edges, pulse width or signal frequency. |
| Output compare | Generate an event or change an output when the counter matches a programmed value. |
| Encoder mode | Count quadrature encoder transitions on supported timers. |
| Trigger output | Trigger ADC or coordinate peripherals without needing a software call for every sample. |

Common categories include basic, general-purpose and advanced-control timers. Exact timer names and capabilities depend on the STM32 model. SysTick is commonly used as a system time base; watchdogs are separate safety mechanisms, not general-purpose application timers.

For a typical up-counting timer:

f_counter = f_TIM / (PSC + 1)

f_update  = f_TIM / ((PSC + 1) * (ARR + 1))

PSC and ARR are register values, so the hardware divides by the value plus one. Check the actual timer input clock, counter mode, repetition counter (if applicable), and any clock division before relying on this calculation.

To calculate a timer tick and the total delay or frequency in an STM32F103C8T6 microcontroller, you need to know your timer input clock frequency ftimer_clk, the Prescaler register value (PSC), and the Auto-Reload Register value (ARR).

1. Clock Tree & Timer Input Frequency

- System Clock (SYSCLK): Typically configured to 72 MHz using an external 8 MHz crystal with the PLL.

- Bus Clocks (APB1 and APB2):

  - TIM1 is connected to APB2 (up to 72 MHz).

  - TIM2–TIM4 are connected to APB1 (up to 36 MHz max for APB1, though the timer clock multiplier doubles it if the APB1 prescaler > 1, resulting in 72 MHz).

- Check your specific timer's input clock ftimer_clk, usually 72 MHz if configured for full speed.

2. Basic Formulas

- Timer Tick Time ttick: The time it takes for the counter to increment by 1.
ttick=PSC+1ftimer_clk

- Timer Clock Frequency fcounter:
fcounter=ftimer_clkPSC+1

- Total Update Event / Overflow Period Tdelay or Tperiod: The timer counts from 0 up to ARR, meaning there are ARR + 1 total counts.
Tdelay=ttick×ARR+1=PSC+1×ARR+1ftimer_clk

3. Step-by-Example Calculation

Suppose you want a 1 ms (0.001 s) tick time and an interrupt or update every 10 ms on an STM32F103C8T6 running at ftimer_clk=72 MHz:

- Calculate Prescaler (PSC) for a 1 ms tick:

  - ttick=1 ms=0.001 s

  - PSC+1=ttick×ftimer_clk=0.001×72,000,000=72,000

  - PSC=72,000-1=71999 (Note: PSC is a 16-bit register, max value is 65535, so a 1 ms tick directly at 72 MHz requires scaling down the APB clock first, or choosing a smaller tick like 1 µs).

- Recalculating with a 1 µs tick ftimer_clk=72 MHz:

  - Target tick time = 1 μs = 0.000001 s

  - PSC + 1 = 0.000001 × 72,000,000 = 72

  - PSC=72-1=71(Configured as 71 in code to divide by 72)

  - Resulting counter clock = 72 MHz72=1 MHz (1 tick = 1 μs).

- Calculate Auto-Reload Register (ARR) for a 1 ms (1000 µs) overflow:

  - ARR+1=Desired DelayTick Time=1000 μs1 μs=1000

  - ARR=1000-1=999 (Configured as 999 in code).

You can automate or double-check these register values using the STM32 Timer Calculator Tool or interactive calculators on DeepBlueMbedded.

## 2.4 Practical timer interrupt demonstration

In CubeMX, enable a timer, choose a prescaler and period, and enable its relevant interrupt in NVIC. Generate code, then start it in the application.

if (HAL_TIM_Base_Start_IT(&htim2) != HAL_OK)

{

Error_Handler();

}

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

{

if (htim->Instance == TIM2)

{

// Keep this short: set a flag or update a counter.

}

}

This is a pattern, not a complete project. The timer must be initialized, its IRQ enabled, and the HAL IRQ handler must call the appropriate HAL timer handler. Prefer setting a flag in the callback and doing heavier work in the main loop/task. Do not use long delays or blocking UART transmissions inside an ISR.

## 2.5 Integrated example: timer + ADC + DMA

Goal: collect samples at a stable hardware-defined rate, store them in RAM, and process blocks without asking the CPU to start every conversion.

| Stage | Role | Teaching points |
| --- | --- | --- |
| Timer | Creates a regular trigger at the selected sampling rate. | Timer clock, PSC/ARR, trigger output selection. |
| ADC | Converts selected analog input(s) into digital codes. | Channel/rank, sample time, resolution, trigger source, conversion time. |
| DMA | Moves conversion results into a RAM buffer. | Request mapping, width, memory increment, circular/normal mode. |
| Callback/IRQ | Notifies firmware about half/full buffer or errors. | Keep callback short; coordinate buffer ownership. |
| Application | Scales, filters, logs or transmits measurements. | Processing must keep up with the sample rate. |

Conceptual data flow:

Timer update/trigger

↓

ADC conversion

↓

DMA writes sample into RAM buffer

↓

Half-transfer / transfer-complete event

↓

Application processes completed samples

Typical configuration outline (exact labels and APIs vary by STM32 family):

Configure the analog input pin and ADC channel.

Set the ADC external trigger to the chosen timer event.

Configure DMA for ADC data register → RAM, with peripheral increment disabled and memory increment enabled.

Choose normal or circular mode and an appropriate buffer size.

Configure the timer's update/trigger event and calculate the sample rate.

Start DMA-based ADC acquisition and start the timer in the correct order for the selected HAL/family.

Use half/full callbacks to process only the region that DMA has finished writing.

Validate with a known voltage, a changing potentiometer, or a signal generator and check for missed/overwritten blocks.

COMMON FAILURE If the ADC is configured for software start while the design expects a timer trigger, sampling will not follow the timer. Also verify that the selected timer event is actually routed to the ADC on that MCU.

## Part 2 task

Configure a periodic timer and acquire an analog sensor value. First demonstrate a simple approach (polling or interrupt), then explain what changes when the timer triggers ADC conversions and DMA fills a buffer.

# PART 3 — UART, SPI and I²C

Suggested duration: 35–40 minutes.

## 3.1 UART / USART

UART is asynchronous serial communication. It commonly uses TX and RX plus a shared ground. Both ends must agree on baud rate, data bits, parity and stop bits. USART peripherals may also support synchronous modes on some MCUs.

Cross TX of one device to RX of the other; connect grounds. Check voltage levels before wiring.

Configure baud rate, word length, parity, stop bits and flow control.

Use a USB-to-UART adapter or supported debugger virtual COM port and a serial terminal.

Polling calls are easy to demonstrate; interrupt/DMA APIs are useful for long or continuous transfers.

A blocking transmit call waits for completion or timeout and can delay other work.

uint8_t message[] = "Hello STM32\r\n";

HAL_UART_Transmit(&huart1, message, sizeof(message) - 1U, 100U);

## 3.2 I²C

I²C is a synchronous, addressed two-wire bus: SDA carries data and SCL carries the clock. Compatible devices can share the bus when addresses, electrical levels, pull-ups and bus capacitance are handled correctly.

Explain 7-bit addressing, START/STOP conditions, ACK/NACK and register-based reads/writes.

SDA and SCL usually need pull-up resistors; some boards/sensors already include them.

Check bus voltage compatibility and sensor power before connecting hardware.

A common failure is confusing a 7-bit address with the shifted address representation expected by a specific HAL API. Check that family's documentation.

Debug with address scans where appropriate, logic-analyzer traces, and datasheet timing.

## 3.3 SPI

SPI is synchronous and often full duplex. Common signals are SCK, MOSI, MISO and CS/NSS. Each selected peripheral typically has its own chip-select line unless the design uses a special topology.

Set controller/peripheral role, data width, clock polarity (CPOL), clock phase (CPHA), bit order and clock prescaler.

Match SPI mode and timing to the device datasheet.

SPI often offers high throughput, but actual speed depends on device limits, wiring and firmware.

SPI typically uses more signal wires than I²C, especially as the number of devices increases.

## 3.4 Protocol selection

| Factor | UART | I²C | SPI |
| --- | --- | --- | --- |
| Clocking | Asynchronous; agreed baud rate | Synchronous; SCL | Synchronous; SCK |
| Typical wires | TX/RX + ground | SDA/SCL + ground | SCK/MOSI/MISO/CS + ground |
| Multiple devices | Point-to-point common; other topologies need extra design | Addressed shared bus | Often one CS per peripheral |
| Strengths | Simple debug links, GPS, serial modules | Convenient low/moderate-speed sensor bus | High-throughput displays, flash, sensors |
| Trade-offs | Baud/framing agreement | Pull-ups, addresses, bus capacitance | More pins/CS management, mode matching |

Decision exercise — ask students to justify a choice:

GPS module sending position data → UART is common.

Several compatible temperature sensors → I²C may reduce wiring if addresses and bus limits work.

Display needing frequent updates → SPI may be suitable if the display supports it.

External flash memory → SPI or a dedicated memory interface, depending on required speed and MCU support.

Do not claim one protocol is always best or fastest. Use the actual device datasheet and application requirements.

## Part 3 task

Transmit a short UART message to a terminal. If hardware permits, read one sensor over I²C or SPI and show how the peripheral settings map to the sensor datasheet.

# PART 4 — Sensors, ADC and Real-Time Data Acquisition

Suggested duration: 35–40 minutes.

## 4.1 Analog vs. digital sensors

| Output type | Example | Typical STM32 path |
| --- | --- | --- |
| Analog voltage | Potentiometer or analog temperature sensor | ADC input → raw code → scaling/conversion. |
| Digital bus | Digital temperature sensor or IMU | I²C/SPI transaction → decode registers/data. |
| Serial stream | GPS module | UART reception → parser → position/time data. |
| Pulse/frequency | Flow or speed sensor | Timer input capture/counter or GPIO interrupt, based on pulse rate. |

A digital sensor may contain its own ADC internally. “Digital” describes the interface exposed to the MCU, not necessarily the physical sensing method inside the package.

## 4.2 Understanding the ADC

An ADC samples an analog voltage and maps it to a digital code. For an ideal N-bit ADC there are 2^N codes. The maximum code is 2^N − 1.

Approximate ideal code = (Vin / Vref) × (2^N − 1)

Approximate voltage     = ADC_code × Vref / (2^N − 1)

For an ideal 12-bit ADC, codes range from 0 to 4095. With Vref = 3.3 V, an input near 1.65 V gives a code near half scale. Actual results depend on reference accuracy, input range, ADC resolution, sampling time, source impedance, noise, calibration and hardware errors.

SAFETY NOTE Never assume an STM32 ADC pin tolerates 5 V. Check the exact MCU datasheet, pin limits and analog input range. Use appropriate scaling/protection for signals outside the permitted range.

## 4.3 What does real-time data acquisition mean?

It means acquiring data under timing constraints that matter to the application. Key metrics include sample rate, sample interval, jitter, latency, buffer depth, processing time and data loss.

| Metric | Meaning |
| --- | --- |
| Sample rate | Number of samples collected per second, e.g. 1 ksample/s. |
| Sample interval | Time between samples; 1 ksample/s corresponds to 1 ms nominally. |
| Jitter | Variation in actual sample timing. |
| Latency | Time between the physical event/sample and the data being acted on or delivered. |
| Throughput | Rate at which the system can move/process data. |
| Overrun/data loss | New data arrives before the old data is safely read or processed. |

A software delay or busy loop does not guarantee precise sampling. Hardware timer triggers can make sample timing more deterministic.

## 4.4 Buffering and data flow

A buffer stores a sequence of samples so acquisition and processing do not have to happen one sample at a time. DMA can fill a buffer while the CPU processes a different completed region.

Normal DMA: transfer a finite number of items and stop or signal completion.

Circular DMA: reuse the buffer continuously; the CPU must keep up and process completed regions before they are overwritten.

Half-transfer and transfer-complete callbacks can divide a buffer into two processing regions.

Use counters or diagnostics to detect overruns and missed processing deadlines.

Avoid sharing a buffer between ISR/DMA and application code without a clear ownership strategy.

## Part 4 task

Read an analog input, convert the raw code into voltage or a sensor unit, and report the result. Then discuss what must change to sample at a fixed rate and preserve blocks of data.

# PART 5 — Sensor Drivers and Reusable Embedded Software

Suggested duration: 30–35 minutes.

## 5.1 What is a driver?

A sensor driver encapsulates the operations needed to configure and communicate with a particular sensor. It offers a clean API so application code does not need to know every register address, bit field or transaction detail.

Initialize and verify the sensor.

Configure range, sample rate, operating mode or filters.

Read raw measurements and convert them to meaningful units where appropriate.

Return clear status/error codes and handle timeouts.

Document units, valid ranges, blocking behavior and configuration assumptions.

## 5.2 A reusable project structure

Core/

Inc/                 generated or shared headers

Src/                 generated startup/initialization and main code

Drivers/

Sensors/

sensor_name/

sensor_name.h

sensor_name.c

Application/

app.h

app.c

This is an example, not a mandatory folder layout. The key idea is to keep the sensor API and implementation separate from application policy and CubeMX-generated initialization.

## 5.3 Example public interface

#ifndef SENSOR_NAME_H

#define SENSOR_NAME_H

#include <stdint.h>

#include "stm32f1xx_hal.h"  // Replace with the correct family header.

HAL_StatusTypeDef Sensor_Init(I2C_HandleTypeDef *hi2c);

HAL_StatusTypeDef Sensor_Read(I2C_HandleTypeDef *hi2c,

float *measurement);

#endif

Explain the interface:

Include guards prevent duplicate header inclusion.

The public function declarations define what the driver promises to provide.

Passing the I²C handle avoids hiding a specific global peripheral inside the driver.

An output pointer returns the measurement; the status return reports success/failure.

The STM32F1 header is family-specific; change it for another family or hide HAL behind a platform abstraction when broader portability is needed.

## 5.4 Application usage

float temperature_c;

if (Sensor_Read(&hi2c1, &temperature_c) == HAL_OK)

{

// Use temperature_c; its unit should be documented.

}

else

{

// Report, retry, or recover according to the application design.

}

## 5.5 What makes a driver reusable?

A clear API with documented units, ranges and error behavior.

Sensor-specific register details stay inside the driver.

No unnecessary application-specific global variables or assumptions.

Peripheral handles or transport interfaces are passed explicitly where practical.

Communication errors and timeouts are handled consistently.

Initialization, configuration and data reading have distinct responsibilities.

The implementation follows the sensor datasheet and validates device identity/status when supported.

Blocking calls are documented; asynchronous designs are considered for high-rate applications.

The driver is tested independently from the final application where possible.

## Part 5 task

Give students a supported sensor and ask them to implement an initialization function and a measurement function. The application should call the API rather than duplicating register-level transactions.

# PART 6 — RTOS: Why Does a Project Need One?

Suggested duration: 25–30 minutes.

## 6.1 Bare-metal superloop vs. RTOS

A superloop is a simple main loop that calls application functions repeatedly. It is often excellent for small, predictable systems.

while (1)

{

ReadSensors();

ProcessData();

UpdateDisplay();

SendData();

}

The weakness appears when one function blocks or takes a long time: later functions may be delayed. An RTOS provides a scheduler and synchronization mechanisms to organize concurrent activities as tasks. On a single-core MCU, tasks usually share CPU time rather than literally execute in parallel.

| Superloop may fit when… | RTOS may be justified when… |
| --- | --- |
| Few activities and simple timing requirements. | Several independent activities have different timing needs. |
| Functions are short, non-blocking and easy to schedule. | Blocking I/O complicates responsiveness. |
| A small, easy-to-audit codebase is the priority. | Tasks need queues, synchronization, or structured scheduling. |
| Memory/complexity budget is tight. | The complexity benefit outweighs stack, RAM and scheduler overhead. |

## 6.2 Essential FreeRTOS concepts

| Concept | Explanation | Typical use |
| --- | --- | --- |
| Task | A scheduled unit of execution with its own stack/context. | Sensor, processing, communication or UI activity. |
| Scheduler | Chooses which ready task runs according to policy and priorities. | Coordinates CPU time. |
| Priority | Affects task selection; exact behavior depends on scheduling configuration. | Meet responsiveness requirements. |
| Queue | Transfers messages/data safely between tasks. | Send samples from acquisition to processing. |
| Semaphore | Signals an event or coordinates access. | Notify that data is ready. |
| Mutex | Provides mutual exclusion for a shared resource. | Protect a shared bus/resource when appropriate. |
| Software timer | Runs a callback after a tick-based period or expiry. | Noncritical periodic actions; not a replacement for precise hardware sampling. |
| Task notification | Lightweight task signaling/data mechanism in FreeRTOS. | Wake a task or signal a simple event. |

## 6.3 Sensor-monitoring architecture

Acquisition → Queue → Processing → Queue / message → Communication

|                         |

hardware IRQ/DMA          filtering/scaling

A sensible design might use a timer-triggered ADC + DMA for acquisition, then notify a task when a buffer region is ready. A processing task filters/scales the samples, and a communication task sends results. Use a queue or another documented handoff method so data ownership is clear.

RTOS CAUTIONS More tasks do not automatically make a system more real-time. Analyze priorities, execution times, deadlines, queue capacity, stack sizes, interrupt priorities and shared resources. Never call RTOS APIs from an interrupt unless that API is explicitly ISR-safe and the interrupt priority is permitted.

## Part 6 task

Ask students to design a sensor-monitoring system with acquisition, processing and communication responsibilities. They must identify what data crosses each boundary and whether a queue, notification or mutex is actually needed.

# Integrated Engineering Decision Guide

| Requirement | Likely starting point | Questions to verify |
| --- | --- | --- |
| Blink an LED or read a rarely used input | GPIO + superloop/polling | Can the loop remain responsive? Is debounce needed? |
| Respond to a button edge | GPIO EXTI interrupt | How will bounce be handled? Keep ISR short. |
| Generate a stable periodic event | Hardware timer | What is the real timer clock? What jitter is acceptable? |
| Generate a motor/LED control waveform | Timer PWM | Frequency, resolution, polarity, safe startup state? |
| Read an occasional analog sensor | ADC polling or interrupt | Conversion time, sample time, voltage range, calibration? |
| Sample an analog waveform at a fixed rate | Timer-triggered ADC | Trigger route, ADC conversion time, aliasing and sample rate? |
| Collect continuous ADC blocks | Timer + ADC + DMA | Buffer size, half/full callbacks, throughput, cache and overruns? |
| Send debug logs to a PC | UART | Baud/framing, blocking time, buffer strategy? |
| Connect several compatible low-rate sensors | I²C | Addresses, pull-ups, bus capacitance, voltage levels? |
| Move display/flash data at high throughput | SPI or dedicated interface | Device timing, mode, pin count, required bandwidth? |
| Organize multiple semi-independent functions | Superloop first; RTOS if justified | Blocking, deadlines, synchronization and RAM budget? |

## Common debugging checklist

Confirm the exact MCU part number and its datasheet/reference manual.

Check power, common ground, voltage levels, pin mapping and alternate functions.

Verify clocks and peripheral clock enablement.

Check generated initialization and HAL return statuses.

Check NVIC enable/priority and IRQ handler routing.

For DMA, verify request mapping, direction, widths, increments, buffer lifetime and callback behavior.

For timers, calculate using the actual timer input clock, not automatically the CPU clock.

For ADC, check channel, pin analog mode, sample time, reference/range, trigger source and calibration requirements.

For serial buses, check baud/mode/address, wiring, pull-ups/CS, and datasheet timing.

Use breakpoints, watch variables, a serial terminal, oscilloscope or logic analyzer when available.

## Instructor preparation checklist

Verify STM32CubeIDE/CubeMX versions and install the correct MCU support package.

Prepare a known-good .ioc project and a backup copy.

Test build, flash and debug before students arrive.

Prepare a GPIO blink demonstration.

Prepare a timer interrupt demonstration.

Prepare an ADC read and, if hardware supports it, timer-triggered ADC + DMA.

Prepare a UART terminal and known-good cable/adapter.

Choose one sensor, read its datasheet, and verify the wiring and driver.

Prepare a small reusable driver example with header and source files.

Prepare a simple FreeRTOS task/queue example or architecture diagram.

Keep screenshots and source backups in case live hardware fails.

## Quick knowledge check

What does the .ioc file store, and what happens when code is regenerated?

Why can a timer's input clock differ from the CPU clock?

Explain the different jobs of a timer, ADC, DMA and interrupt.

When would polling be preferable to DMA?

What electrical requirements does I²C usually need that UART does not?

Why is a driver API better than duplicating sensor register reads throughout the application?

What problems can a queue solve between RTOS tasks?

What can cause DMA buffer data to be overwritten before processing?

## Closing message for students

THE BIG PICTURE Reliable embedded systems come from choosing the right hardware feature for the requirement, configuring it correctly, understanding the data path, handling timing and errors, and keeping application logic modular. Learn the concepts deeply enough to explain why a design works—not only which buttons to click in CubeMX.